| Parameter | Specification | Compliance (Yes/No) |
| --- | --- | --- |
| Type | Indoor floor-standing digital display-signage kiosk/standee | YES |
| Orientation | Portrait | YES |
| Cabinet material | Mild-steel or superior metallic enclosure | YES |
| Finish | Powder-coated, corrosion-resistant and suitable for professional indoor installation | YES |
| Structure | Stable, anti-toppling, floor-standing construction | YES |
| Access | Lockable rear/service-access panel | YES |
| Cable management | Concealed routing of power and network cables | YES |
| Ventilation | Suitable ventilation and heat dissipation for extended operation | YES |
| Edges | Smooth, finished and free from exposed sharp edges | YES |
| Colour | Black, grey, white or other colour approved by MCL | YES |
| Location type | Indoor | YES |
| Screen size | Approximately 55 inches; diagonal display size between 138.1 cm and 143 cm | YES |
| Display resolution | Minimum 3840 × 2160 pixels, 4K UHD | YES |
| Aspect ratio | Portrait 9:16 | YES |
| Brightness | Minimum 400 nits | YES |
| Viewing angle | Minimum 178 degrees horizontal and vertical | YES |
| Display technology | Commercial-grade LED/LCD or superior | YES |
| Surface | Anti-glare or low-reflection surface | YES |
| Operation | Suitable for extended daily commercial-signage operation | YES |
| Audio | Built-in speakers, wherever supported | YES |
| Content | Images, videos, text, animation and other standard digital-signage formats | YES |

| Processor | ARM-based or equivalent commercial-signage processor | YES |
| --- | --- | --- |
| RAM | Minimum 4 GB | YES |
| Internal storage | Minimum 64 GB | YES |
| Storage type | SSD, eMMC or other suitable non-volatile storage | YES |
| Operating system | Android 11 or later, or equivalent commercial digital-signage operating system | YES |
| Media player | Integrated professional digital-signage media player | YES |
| Auto-start | Standee and signage application shall restart automatically after restoration of power | YES |
| Local storage | Content shall be capable of being downloaded and stored locally for continued playback during temporary internet interruption | YES |
| Clock | Automatic date and time synchronisation | YES |
| Firmware | Secure firmware and software-update mechanism | YES |

| Parameter | Compliance (Yes/No) |
| --- | --- |
| Secure web-based login | YES |
| Facility to disable or remove users | YES |
| Upload of images and videos | YES |
| Standard image formats such as JPEG and PNG | YES |
| Standard video formats such as MP4 | YES |
| Text messages and tickers, where available | YES |
| Portrait-oriented content | YES |
| Individual and bulk content publication | YES |
| Playlist creation | YES |
| Content Preview & Content Manage | YES |
| Common content across all locations | YES |
| Different content for different locations | YES |
| Automatic content download to standees | YES |
| Date-wise scheduling & Time-wise scheduling | YES |
| Day-of-week scheduling | YES |
| Start and Expiry Dates & Automatic removal of Expired Content | YES |
| Repeating Schedules & Playlist Scheduling | YES |
| Additional Marked Features / Requirements Not Available |  |
| Remote CMS operation: Remote content publication, scheduling, monitoring and emergency override. | NO |
| Role-based access control. | NO |
| User roles: Administrator, content publisher and viewer roles. | NO |
| User-wise access restriction: User-wise access restriction. | NO |
| Password-management controls. | NO |
| User login & content activity records: Record of user login and content activities. | NO |
| Audit trail: Audit trail of configuration and publication changes. | NO |
| Online/offline status monitoring. | NO |
| Device-health monitoring. | NO |
| Storage-status monitoring. | NO |
| Last-connected status. | NO |
| Remote restart, wherever technically supported. | NO |
| Remote software/firmware update. | NO |
| Screenshot or proof-of-display facility, wherever supported. | NO |
| Alerts for offline or malfunctioning devices. | NO |
| Local playback after content synchronisation. | NO |
| Advance scheduling of event content. | NO |
| Restoration of generic content after expiry of event-specific content. | NO |
| Emergency content override: Authorised administrator can publish urgent content to all, selected building/group, or individual standees. | NO |
| Emergency override withdrawal: Emergency content overrides the normal playlist and is withdrawn only by an authorised user or on expiry of the defined schedule. | NO |
| Device-status reports. | NO |
| Content-publication history. | NO |
| User-activity records. | NO |
| Content expiry records. | NO |
| Proof of publication or playback, wherever supported. | NO |
| Offline-device history. | NO |
| Export of reports in commonly used formats. | NO |
| Testing of offline playback. | NO |
| Testing of emergency override. | NO |
| Emergency content override. | NO |
| Acceptance: device monitoring: Device online/offline monitoring. | NO |
| Acceptance: local playback: Local playback during temporary internet interruption. | NO |
| Acceptance :Report generation. | NO |
| Emergency override. | NO |
| Training: device-status monitoring | NO |
| Training: report generation. | NO |